Specifications for Archival Shelving in Localities

All components of shelving to be modular and conform to the following specifications:

Finish: To be baked enamel or powder coated, all surfaces cleaned and phosphate washed, and putty or sand in color.

Shelves: Twenty two (22) gauge steel, cold-rolled box edge type (front and back). Front and rear box beam reinforcements are one and one-fourth (1-1/4) inches high by fifteen/thirty-two (15/32) inches wide and welded to shelf on six (6) centers. The corners are to be lapped and welded for rigidity. Each shelf shall support a minimum load of 500 pounds. Shelves to hold one layer of six (6) standard size file

boxes weighing forty to fifty pounds each. Each unit will have six (6) shelves and a cap. Shelf sizes will be 42" wide by 30" deep.

Shelf Clips: Fourteen (14) gauge, hot-rolled, one piece construction with two (2) tabs to engage the seven-sixteen (7/16) square holes of the upright on one (1) inch centers. Minimum of four (4) shelf clips per shelf fully rated for all potential shelf loads.

Uprights: Angle posts one (1) inch by two and one-sixteenth (2-1/16) inch thirteen (13) or fourteen (14) gauge, cold-rolled steel to form “T” uprights. Posts shall be punched on one (1) or one and one-half (1-1/2) inch centers with pairs parallel to slots eleven/sixteenth (11/16) inch long, with one slot keyhole-shaped for bolting cross braces and accessories to uprights. All posts shall be one-piece posts.

Opened Uprights: To consist of two (2) sixteen (16) gauge, cold-rolled “T”s, one and one-half  (1-1/2) inches by two and one-eight (2-1/8) inches by one-eighth (1/8) inch with two pairs of one (1) inch twelve (12) gauge band cross-braces on all uprights.

Cross-Braces: One (1) inch twelve (12) gauge band cross-braces. Minimum of two (2) pairs of lateral cross-braces per unit and two (2) pairs of upright assembly cross-braces on each upright.

Shim Plate: Twenty two (22) gauge steel and less than two and one-half (2-1/2) inch by two and one-half (2-1/2) inch in size to be used where needed.